- SECTION 10440

- INTERIOR SIGNAGE

Display hidden notes to specifier. (Don't know how? Click Here)

# GENERAL

## SECTION INCLUDES

### Interior Signage

### Specialty Signs & Graphics

## RELATED SECTIONS

### Section 09200 - Plaster and Gypsum Board.

### Section 09260 - Gypsum Board Systems.

### Section 09510 - Acoustical Ceilings.

### Section 09900 - Painting.

### Section 09960 - Vinyl Wall Covering.

## REFERENCES

### 2010 ADA Standards for Accessible Design

### ICC/ANSI A117.1 - Accessible and Useable Buildings and Facilities; 2003

### USATBCB - Americans with Disabilities Act (ADA), Accessibility Guidelines for Buildings and Facilities (ADAAG).

## SUBMITTALS

### Submit under provisions of Section 01300.

### Shop Drawings: Shop drawings containing plans, elevations, sections and details for all work in this section with letter style, general layout for each sign type, sizes, edge and corner treatment and mounting methods shown.

### Selection Samples: For each finish specified, two complete sets of color chips representing manufacturer's standard choices for color(s), pattern(s) and finishe(s).

### Message Schedule: Architect to provide schedule of messages for all signs.

## QUALITY ASSURANCE

### Manufacturer Qualifications: Manufacturer shall have five years experience manufacturing and fabricating products of similar type and scope as those specified in this section.

### Installer Qualifications: Minimum five years documented experience in work of this section.

### Single Source Requirements: Obtain all products in this section from a single supplier.

### Mock-Up: As requested by architect, provide a mock-up of select sign types for evaluation of finishes and application workmanship.

#### Finishes designated in shop drawing and selected by Architect.

#### Do not proceed with remaining work until workmanship, color and finish are approved by Architect.

#### Refinish mock-up area as required to produce acceptable work.

## DELIVERY, STORAGE AND HANDLING

### Comply with manufacturer's ordering instructions and lead-time requirements to avoid construction delays. Thoroughly inspect products upon receipt.

### Deliver products in manufacturer's original, unopened, undamaged containers and packaging with labels clearly identifying product name and manufacturer intact.

### Store products protected from weather, temperature and other harmful conditions in accordance with manufacturer's instructions.

### Protect materials during handling and installation to prevent damage.

## PROJECT CONDITIONS

### Maintain environmental conditions (temperature, humidity, and ventilation) within limits recommended by manufacturer for optimum results. Do not install products under environmental conditions outside manufacturer's absolute limits.

## WARRANTY

### Provide written documentation of manufacturer’s warranty

#### Warranty must guarantee interior signs for the life of the building.

# PRODUCTS

## MANUFACTURERS

### Acceptable Manufacturer: Century Sign Builders | (800) 279-2904 | hello@csbsigns.com | www.csbsigns.com

### Substitutions: Not permitted.

## INTERIOR SIGNAGE

### [SIGN SYSTEM]  by Century Sign Builders

#### [main sign system info goes here]

### Specialty Signs & Graphics [note this heading is added to Sec 1.1]

#### Vinyl Lettering Signs

##### Provide signs made of individual letters made of cut vinyl graphic film applied directly to mounting surface (glass, smooth wall, doors, etc.).

##### Material & Colors:

###### Opaque & Translucent: To be constructed of high performance cast vinyl graphic film products such as 3M Scotchcal or equal with a 5 year or greater warranty.

###### Frosted Glass Effect: To be constructed of  3M Scotchcal ElectroCut Graphic Film Frosted Crystal, 7725SE-324.

##### All lettering must be computer cut from professional quality artwork.  No hand cut vinyl film shall be installed.

##### Signs shall be cut from sufficiently large rolls of material to minimize seams or joining of material to create one sign.

##### Signs shall be provided in the sizes & quantities indicated on the shop drawings.

##### Installation method:

###### Signs shall be installed free of bubbles, wrinkles or other anomalies.

###### Provide signs as either front applied (first surface) or reverse applied (second surface) as required.

#### Evacuation Plan Signs

##### General Design: Evacuation plan holder signs shall be constructed to hold a printed graphic insert (evacuation plan) by forming a space between two layers of material (lens & backer) where the insert is contained.  Decorative borders will be added to the top and bottom edges.

##### Backer material: Non-Glare plastic sign panel

###### Colored non-glare acrylic multi-polymer by Rowmark plastics.

###### Products with painted or otherwise applied coloration method are not acceptable.

###### Finishes and color as per approved shop drawings.

##### Lens or cover material: lens and covers shall be constructed using 0.0625” (322-101) clear single-ply non-glare acrylic multi-polymer (Rowmark).

##### Decorative Border Material: Non-Glare plastic sign panel

###### Colored non-glare acrylic multi-polymer by Rowmark plastics.

###### Products with painted or otherwise applied coloration method are not acceptable.

###### Bond to sign plaque using 3M Scotch 467HP adhesive.

###### Border thickness as per approved shop drawings.

###### Finishes and color as per approved shop drawings.

##### Printed graphic inserts: Printed inserts will be created using a satin-coated, tear-resistant, rigid PVC media with eco-solvent waterfast & UV stable inks.

###### Printed background inserts must be manufactured in color managed workflow with the following capacities:

###### All printing must be done using a profiled printer with transmissible ICC profile.

###### All approved colors used in final design must have LAB values recorded and submitted to architect owner for future reference and duplication.

###### Printing must be performed on calibrated printer such that future orders of insert can be reproduced within 5 Delta E of recorded LAB values.

##### Installation method:

###### Wall mounted signs: Signs shall be mounted using double-sided vinyl foam tape (1/16” thickness), silicon adhesive, or mechanical anchors as per the approved shop drawings.

#### Dimensional Letters Signs

##### General Design:  Dimensional letters signs shall be created using individual metal lettering and logos mounted together to form signs.

##### Base Material: Individual letters and logos shall be cut from solid sheets of metal to create solid forms.

###### Base metal: As per approved shop drawings

##### Finish: Signs shall have a mechanical finish (grained or polished) and an applied finish (paint, clear coat or anodized) as per the approved shop drawings.

##### Signs shall be formed with edges which are smooth and free from defects.

##### Installation method:

###### Signs shall be mounted using metal studs on the rear of the sign which are inserted into holes prepared with silicon adhesive in the mounting surface.

###### Sign installation shall only be performed by qualified installer approved by the manufacturer.

##### Letter Sizes: As per approved shop drawings.

#### Dedication Plaque

##### General Design: Dedication plaque signs shall be formed from a single metal panel on which letters and graphics are created

##### Base Material: Form from a single metal (Aluminum, Bronze, Steel) panel as per the approved shop drawings.

##### Lettering Graphic Method: Etched, engraved or cast as per the approved shop drawings.

##### Finish: Signs shall have a mechanical finish (grained, pebble or polished) and an applied finish (paint, clear coat or anodized) as per the approved shop drawings.

##### Signs shall be formed with edges which are smooth and free from defects.

##### Installation method:

###### Signs shall be mounted using metal studs on the rear of the sign which are inserted into holes prepared with silicon adhesive in the mounting surface.

###### Sign installation shall only be performed by qualified installer approved by the manufacturer.

##### Size: As per approved shop drawings.

# EXECUTION

## EXAMINATION

### Examine signage for defects prior to installation.  Do not install damaged signage.

### Inspect conditions of installation areas and other conditions which may affect installation of signage to ensure that conditions are suitable for installation.

### Do not begin installation until installation areas are within manufacturer's specified tolerances and have been prepared in accordance with manufacturer's instructions.

### If installation area preparation is the responsibility of another installer, do not proceed with installation. Notify Architect of unsatisfactory preparation immediately.

### Commencement of work is deemed as acceptance of installation conditions.

## PREPARATION

### Verify mounting heights and locations for signage will comply with specified requirements.

### Clean mounting locations of dirt, dust, grease or similar conditions that would prevent proper installation.

### Prepare surfaces using the methods recommended by the manufacturer for achieving the best result for the substrate under the project conditions.

### Verify completion of other installation conditions needed for sign installation including backing materials, reinforcement, electrical and data.

## INSTALLATION

### Install in accordance with manufacturer's instructions.

### Locate signs in accordance with approved shop drawings and project requirements.

## CLEANING, PROTECTION AND REPAIR

### Protect installed products until completion of project.

### Touch-up, repair or replace damaged products before Substantial Completion.

## Training & Closeout

### Provide manufacturer’s written warranty and cleaning/maintenance instructions.

### Provide digital templates for end-user updatable inserts.

### Provide necessary tools and source for consumables for end-user updateable inserts.

END OF SECTION